Safe Work Method Statement Residential New Build Construction — All High-Risk Work Activities Doc No: G5-SWMS-001 ¦ Version: 1.0 ¦ Issue: May 2026

IMPORTANT: This SWMS must be completed, signed, and workers inducted before commencing any high-risk construction work. Review and update for each new project.

# SECTION 1 — PROJECT & BUSINESS DETAILS

| Business Name |  |
| --- | --- |
| ABN |  |
| Licence Number |  |
| Principal Contractor |  |
| Project Address |  |
| Project Description |  |
| Estimated Start Date |  |
| Estimated Duration |  |
| Site Supervisor / WHS Officer |  |
| Emergency Contact Name & Phone |  |
| Nearest Hospital / Medical Centre |  |

# SECTION 2 — HIGH-RISK CONSTRUCTION WORK COVERED

This SWMS covers the following high-risk construction work activities as defined in the Work Health and Safety Regulations, Schedule 18:

- Work involving a risk of a person falling more than 2 metres

- Work on a telecommunications tower

- Demolition of load-bearing structure

- Work in or near a shaft/trench deeper than 1.5m, or on a tunnel

- Work involving asbestos

- Work involving structural alterations requiring temporary support

- Work carried out in or near a confined space

- Work carried out in or near a trench or shaft deeper than 1.5 metres

- Work carried out on or near pressurised gas distribution mains or piping

- Work carried out on or near energised electrical installations or services

- Work carried out in an area where there is movement of powered mobile plant

- Work carried out in areas containing or adjacent to explosives

- Work on or near roads or traffic corridors in use by traffic other than pedestrians

- Work carried out at a workplace in a remote or isolated area

- Work carried out in an area with artificial extremes of temperature

- Work carried out in or around water or other liquids involving risk of drowning

# SECTION 3 — RISK ASSESSMENT & CONTROL MEASURES

Risk ratings: Extreme (E) = 15–25 | High (H) = 8–14 | Medium (M) = 4–7 | Low (L) = 1–3

| Activity / Task | Hazard / Risk | Initial Risk | Control Measures (Hierarchy of Controls Applied) | Residual Risk |
| --- | --- | --- | --- | --- |
| Working at heights (scaffolding, roofing, formwork) | Falls from height | High | Scaffold erected by licensed scaffolder. Full perimeter edge protection. Safety harness at >2m. No working from ladders above 2m for sustained work. | Medium |
| Manual handling – lifting heavy materials (timber, concrete, roofing sheets) | Musculoskeletal injury, crush injury | High | Mechanical aids (pallet jacks, crane) used where possible. Team lifts for items >16kg. Workers trained in manual handling techniques. | Low |
| Power tools and hand tools (circular saws, nail guns, grinders) | Laceration, puncture, eye/hearing injury | High | Guards in place and checked before use. PPE: safety glasses, hearing protection, gloves. No tool modifications. Tools inspected pre-use. | Low |
| Excavation and trenching (footings, services) | Cave-in, striking underground services, falls into excavation | High | Dial Before You Dig completed. Trial holes dug manually near services. Shoring or battered sides for trenches >1.5m. Barriers around open excavations. | Medium |
| Concrete works (pouring, vibrating, finishing) | Cement burns, eye injury, noise | Medium | Waterproof gloves and boots worn. Eye protection mandatory. Wash facilities on site. Barrier cream applied. Hearing protection when vibrating. | Low |
| Framing and structural work (wall frames, roof trusses) | Falls, crush from falling members, struck-by | High | Crane lifts supervised by licensed operator. Tag lines used to guide loads. Exclusion zones under crane. Bracing immediately after placement. | Medium |
| Electrical work (temporary power, rough-in) | Electrocution, electric shock, fire | High | Licensed electrician only. RCDs on all temporary supplies. No work on live circuits. Electrical equipment tested and tagged. | Low |
| Plumbing (rough-in, drainage, gas fitting) | Injury from power tools, gas leak, exposure to wastewater | Medium | Licensed plumber for gas work. Gas leak test before commissioning. Appropriate PPE for sewage/drainage work. | Low |
| Hot works (welding, grinding, cutting) | Burns, fire, eye injury, fumes | High | Hot works permit required. Fire extinguisher within 5m. Fire watch for 30 minutes post-work. Welding screen. Respiratory protection if confined space. | Medium |
| Hazardous materials (asbestos, lead paint on renovation sites) | Asbestos-related disease, lead poisoning | Extreme | Asbestos register reviewed before demolition. Licensed removalist for friable asbestos. Air monitoring if required. Waste disposed per EPA guidelines. | Low |
| Housekeeping and site access | Slips, trips, falls on cluttered site | Medium | Daily housekeeping. Clear walkways maintained. Adequate lighting. Visitor PPE and induction required. | Low |
| Weather conditions (heat, storm, lightning) | Heat stress, struck by lightning, wind-affected materials | Medium | Heat management plan in summer. Cease outdoor work in storms. Secure loose materials. Sun protection for outdoor workers. | Low |

# SECTION 4 — PERSONAL PROTECTIVE EQUIPMENT (PPE) REQUIREMENTS

| PPE Item | Required For |
| --- | --- |
| Hard hat / safety helmet | All workers on site at all times |
| High-visibility vest | All workers on site at all times |
| Safety footwear (steel-capped boots) | All workers on site at all times |
| Safety glasses / eye protection | All power tool use, concrete work, grinding |
| Hearing protection (Class 4+) | Power tools, nail guns, compressors, vibrating equipment |
| Gloves (general purpose) | Manual handling, concrete work, sharp materials |
| Respiratory protection (P2 minimum) | Cutting, sanding, dusty environments, asbestos suspect areas |
| Fall arrest harness (AS/NZS 1891) | Working at height >2m where other fall protection not feasible |
| Sun protection (hat, SPF50+ sunscreen) | All outdoor workers — reapply every 2 hours |

# SECTION 5 — EMERGENCY PROCEDURES

| Emergency services | 000 |
| --- | --- |
| Nearest hospital |  |
| Site first aid officer |  |
| First aid kit location |  |
| Muster point |  |
| Incident reporting procedure | Report to site supervisor immediately. Complete Incident Report form G5-INC-007 within 24 hours. Notify WHS regulator if notifiable incident. |

# SECTION 6 — WORKER INDUCTION & SIGN-OFF

All workers must be inducted into this SWMS before commencing work. By signing below, workers confirm they have read, understood, and agree to follow the control measures in this SWMS.

| Full Name | Company / Trade | Date Inducted | Signature | Induction Conducted By |
| --- | --- | --- | --- | --- |

# SECTION 7 — SWMS REVIEW RECORD

This SWMS must be reviewed if: conditions change, a near miss or incident occurs, or a worker raises a concern. Record reviews below.

| Date | Reviewed By | Reason for Review | Changes Made |
| --- | --- | --- | --- |