Safe Work Method Statement Timber Framing and Trusses — Residential Construction Doc No: G5-SWMS-011 ¦ Version: 1.0 ¦ Issue: May 2026

Complete project details, induct all workers, and obtain signatures BEFORE commencing work.

# PROJECT DETAILS

| Business Name |  |
| --- | --- |
| Project Address |  |
| Site Supervisor |  |
| Date |  |
| Estimated Duration |  |

# RISK ASSESSMENT & CONTROL MEASURES

| Task / Activity | Hazard | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Setting out and measuring | Manual handling injury, inaccurate measurements | Medium | Use measuring aids. Two-person lifts for long timbers. Check drawings before cutting. | Low |
| Cutting timber (circular saw, mitre saw) | Laceration, eye injury, noise | High | Guards in place. Push stick for small cuts. Eye and ear protection mandatory. PPE: safety glasses, hearing protection. | Low |
| Erecting wall frames (manual lifting) | Falls, musculoskeletal injury, crush | High | Crane or forklift for heavy frames where possible. Team lifts. Brace frames immediately after erection. Exclusion zone during lift. | Medium |
| Nailing (nail gun / hand nailing) | Puncture wound, eye injury | High | Never point nail gun at person. Finger off trigger when not nailing. Safety glasses. Check nail gun safety mechanism before use. | Low |
| Working at height (top plate, roof trusses) | Fall from height | High | Edge protection at perimeter. Safety harness if edge protection not possible. Scaffold or elevated work platform. No work from ladders for sustained tasks. | Medium |
| Installing roof trusses (crane lift) | Crush injury, truss collapse during erection | High | Licensed crane operator. Dogman / rigger for lifts. Tag lines to guide. Permanent bracing fitted immediately. Exclusion zone below lift. | Low |
| Handling treated timber (CCA) | Skin/eye irritation from preservative chemicals | Medium | Gloves and eye protection when cutting or handling. Wash hands before eating. Do not burn treated timber offcuts. | Low |

# PPE REQUIREMENTS

- Hard hat

- Hi-vis vest

- Steel-capped boots

- Safety glasses

- Gloves

# WORKER SIGN-OFF

| Full Name | Company | Date | Signature |
| --- | --- | --- | --- |