Safe Work Method Statement Concreting — Residential Construction Doc No: G5-SWMS-012 ¦ Version: 1.0 ¦ Issue: May 2026

Complete project details, induct all workers, and obtain signatures BEFORE commencing work.

# PROJECT DETAILS

| Business Name |  |
| --- | --- |
| Project Address |  |
| Site Supervisor |  |
| Date |  |
| Estimated Duration |  |

# RISK ASSESSMENT & CONTROL MEASURES

| Task / Activity | Hazard | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Setting up formwork | Manual handling injury, struck by falling formwork | High | Inspect formwork components before use. Team lifts. Ensure formwork is braced and supported. Exclusion zone during stripping. | Medium |
| Mixing/handling concrete (wet) | Cement burns to skin and eyes, slips on wet surfaces | High | Waterproof gloves and rubber boots. Eye protection. Barrier cream on exposed skin. Wash skin immediately if exposed. Non-slip footwear. | Low |
| Concrete pump/chute operation | Struck by chute, slip/fall | High | Trained operator only. Clear communication between pump operator and placers. Barriers around pump. | Low |
| Vibrating concrete | Vibration white finger (prolonged use), noise | Medium | Limit continuous vibrator use to 10 minutes. Rotate workers. Hearing protection. Anti-vibration gloves. | Low |
| Finishing (floating, trowelling) | Knee strain, prolonged kneeling on wet concrete | Medium | Knee pads. Knee boards to distribute weight. Regular breaks. | Low |
| Stripping formwork | Falls, crush from released formwork members | High | Only strip when concrete has achieved adequate strength. Work from scaffold — no standing on form panels. Nails removed / bent over immediately. | Low |
| Hot/cold weather concreting | Heat stress or delayed set/cracking | Medium | Pour in cool part of day in summer. Sun protection. Adequate hydration. Curing compound or wet hessian in hot/dry conditions. | Low |

# PPE REQUIREMENTS

- Hard hat

- Hi-vis vest

- Steel-capped boots

- Safety glasses

- Gloves

# WORKER SIGN-OFF

| Full Name | Company | Date | Signature |
| --- | --- | --- | --- |