Safe Work Method Statement Roofing — Residential Construction Doc No: G5-SWMS-013 ¦ Version: 1.0 ¦ Issue: May 2026

Complete project details, induct all workers, and obtain signatures BEFORE commencing work.

# PROJECT DETAILS

| Business Name |  |
| --- | --- |
| Project Address |  |
| Site Supervisor |  |
| Date |  |
| Estimated Duration |  |

# RISK ASSESSMENT & CONTROL MEASURES

| Task / Activity | Hazard | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Accessing roof (ladder use) | Fall from ladder, ladder slip | High | Ladder extends 1m above access point. Foot of ladder secured or footed. 4:1 angle. No carrying loads up ladder — use rope or hoist. | Medium |
| Working on roof surface (walking, nailing) | Fall from roof edge, fall through fragile surface | High | Full perimeter edge protection or safety mesh required. Safety harness if edge protection not possible. Wear non-slip footwear. Do not walk on fragile materials (old fibro, Zincalume unsupported). | Medium |
| Installing metal roofing sheets | Laceration from sheet edges, wind-lifted sheet | High | Heavy gloves for handling sheet metal. Work with wind — never allow unsecured sheets in wind. Anchor sheets as placed. | Low |
| Installing ridge capping | Fall — working at highest point | High | Safety harness anchored to ridge fixture or static line. Two workers minimum. No work in high winds. | Medium |
| Fixing sarking and insulation under roofing | Fall through ceiling if trusses slipped | Medium | Step on trusses only, not between. Temporary decking boards. Safety harness. | Low |
| Using nail gun / screw gun on roof | Eye injury, puncture wound, tool drop from height | High | Eye protection. Tether tools to harness. Clear area below of persons during work. | Low |
| Hot weather work | Heat stress, burns from hot metal | High | Work in cooler part of day. Sun protection. Adequate hydration. Allow metal to cool before handling bare-handed. Seek shade during breaks. | Low |

# PPE REQUIREMENTS

- Hard hat

- Hi-vis vest

- Steel-capped boots

- Safety glasses

- Gloves

# WORKER SIGN-OFF

| Full Name | Company | Date | Signature |
| --- | --- | --- | --- |