Safe Work Method Statement Painting — Residential Construction Doc No: G5-SWMS-014 ¦ Version: 1.0 ¦ Issue: May 2026

Complete project details, induct all workers, and obtain signatures BEFORE commencing work.

# PROJECT DETAILS

| Business Name |  |
| --- | --- |
| Project Address |  |
| Site Supervisor |  |
| Date |  |
| Estimated Duration |  |

# RISK ASSESSMENT & CONTROL MEASURES

| Task / Activity | Hazard | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Surface preparation (sanding, scraping) | Dust inhalation, lead paint exposure, eye injury | High | P2 respirator for sanding. Check for lead paint on pre-1970 surfaces — lead paint test kit. Safety glasses. Dust extraction on power sanders. | Low |
| Applying chemical strippers / solvents | Chemical burns, inhalation of vapours, fire risk | High | Check SDS before use. Nitrile gloves. Eye protection. Adequate ventilation — open windows/doors. No smoking or ignition sources nearby. | Medium |
| Working on ladders (painting ceilings, high walls) | Fall from ladder | High | Appropriate ladder for task. Ladder secured or footed. Extension pole for roller to minimise ladder time. No overreaching. | Medium |
| Painting high internal areas (scaffold) | Fall from scaffold | High | Scaffold erected by licensed scaffolder. Handrails in place. No working from edge of scaffold platform. | Low |
| Spray painting (airless spray) | Injection injury, inhalation of overspray, fire | High | Never point spray gun at person. Full-face respirator (OV/P2). Cover and mask adjacent surfaces. Adequate ventilation. No airless spray near ignition sources. | Medium |
| Applying exterior paints in sun | Heat stress, UV exposure | Medium | Work in shade where possible. Sun protection. Hydration. Paint temperature on surfaces — do not apply in extreme heat. | Low |
| Cleaning brushes/equipment with solvents | Skin absorption, fire risk | Medium | Nitrile gloves. Dispose of solvent-soaked rags in sealed metal bin — spontaneous combustion risk. Ventilated area. | Low |

# PPE REQUIREMENTS

- Hard hat

- Hi-vis vest

- Steel-capped boots

- Safety glasses

- Gloves

# WORKER SIGN-OFF

| Full Name | Company | Date | Signature |
| --- | --- | --- | --- |