Safe Work Method Statement Plasterboard Linings — Residential Construction Doc No: G5-SWMS-015 ¦ Version: 1.0 ¦ Issue: May 2026

Complete project details, induct all workers, and obtain signatures BEFORE commencing work.

# PROJECT DETAILS

| Business Name |  |
| --- | --- |
| Project Address |  |
| Site Supervisor |  |
| Date |  |
| Estimated Duration |  |

# RISK ASSESSMENT & CONTROL MEASURES

| Task / Activity | Hazard | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Manual handling of plasterboard sheets | Back/shoulder injury from heavy awkward sheets | High | Two-person lift for 1200x2400 sheets (weight ~23kg). Plasterboard lifter / panel lift for ceiling sheets. Trolley for moving stacks. | Low |
| Cutting plasterboard (score and snap / jigsaw) | Eye injury from dust, laceration, noise | Medium | Safety glasses. Dust mask (P2) when scoring. Jigsaw: eye and hearing protection. Deburr cut edges. | Low |
| Ceiling installation using panel lift / stilts | Fall, crush from dropped sheet, stilt fall | High | Use panel lift — do not hold sheets overhead. If stilts used — trained user only. No stilts on scaffold. Overhead exclusion zone. | Medium |
| Fixing (screwgun) | Eye injury from flying fixings, noise | Medium | Eye protection. Hearing protection in enclosed rooms. Inspect bits and fixings before use. | Low |
| Stopping / plastering | Skin irritation from plaster compounds | Low | Gloves for prolonged contact. Wash skin after work. Adequate ventilation for spray-applied compounds. | Low |
| Working in confined / poorly-ventilated areas | Accumulation of dust, heat stress | Medium | Adequate ventilation — open windows. P2 dust mask. Regular breaks. Hydration. | Low |

# PPE REQUIREMENTS

- Hard hat

- Hi-vis vest

- Steel-capped boots

- Safety glasses

- Gloves

# WORKER SIGN-OFF

| Full Name | Company | Date | Signature |
| --- | --- | --- | --- |